# Assignment – 3

- List 5 common BIOS or POST error messages you might see when starting a PC and write what hardware or configuration issue each one usually points to.

| No. | BIOS/POST Error Message | Usually Indicates |
| --- | --- | --- |
| 1. | CMOS Checksum Error | CMOS battery is weak/dead or BIOS settings are corrupted. |
| 2. | Memory Error / RAM Error | RAM is not properly seated, faulty, or incompatible. |
| 3. | CPU Fan Error | CPU fan is disconnected, not spinning, or incorrectly connected. |
| 4. | Keyboard Error / Keyboard Not Detected | Keyboard is disconnected, faulty, or not detected by the motherboard. |
| 5. | No Boot Device Found | HDD/SSD is not detected, disconnected, or the boot configuration is incorrect. |

- Given this scenario: On boot, a PC beeps 3 times in a repeating pattern and does not display anything on the screen. Research and write what this POST beep code typically means for an AMI BIOS, and suggest two troubleshooting steps.

3 Beeps on Boot: AMI BIOS

Meaning: For classic AMI BIOS, 3 short beeps typically indicate a base memory/RAM read-write failure, meaning the system is having trouble accessing its RAM during POST.

Troubleshooting Steps

- Power off the PC and disconnect the power cable.

- Remove and reseat the RAM modules firmly in their DIMM slots.

- Test one RAM stick at a time to identify a faulty module.

- If the problem continues, try a known-good compatible RAM module.

- Check the motherboard manual because beep patterns can vary by motherboard and BIOS/UEFI implementation.

- Access the BIOS setup utility on any available PC or virtual machine, take a screenshot of the main BIOS screen, and identify the motherboard manufacturer and BIOS version.

BIOS Setup Utility

- Restart the PC.

- During startup, repeatedly press Delete, F2, F10, or Esc depending on the motherboard.

- The BIOS/UEFI Setup Utility will open.

- Open the Main/System Information page.

- Find Motherboard Manufacturer or System Manufacturer.

- Find BIOS Version or BIOS Revision.

- Take a screenshot of the main BIOS screen showing these details.

- Add the screenshot to the assignment.

| Item | Details |
| --- | --- |
| Motherboard Manufacturer | Dell Inc. |
| BIOS Version | Dell Inc. 1.56.0 |
| BIOS Date | 30-06-2026 |
| System Model | Dell Latitude 5420 |
| BIOS Mode | UEFI |

- You are given a screenshot of a POST error message that says 'CMOS checksum error – Defaults loaded.' Explain what this means and describe how you would resolve this issue in a real system.<br><br><em><strong>Hint:</strong> Think about the CMOS battery and BIOS settings reset.</em>

CMOS Checksum Error – Defaults Loaded

Meaning:

- The BIOS/UEFI detected that the stored CMOS settings are invalid or corrupted.

- It loads the default BIOS settings so the PC can continue starting.

- A weak or dead CMOS battery is a common cause.

- It can also happen after a BIOS reset or incorrect BIOS settings.

Steps to Resolve

- Enter BIOS/UEFI during startup by pressing the required key, such as Delete, F2, or F10.

- Select Load Setup Defaults / Optimized Defaults.

- Check the date and time and correct them if necessary.

- Save the BIOS settings and restart the PC.

- If the error appears again, turn off and unplug the PC.

- Replace the CMOS/CR2032 battery with a compatible new battery.

- Start the PC and enter BIOS again.

- Set the required settings, such as boot order, and save the changes.

- Restart and check whether the error has disappeared.