# Assignment – 4

- Open your desktop or laptop, locate the RAM slots, and remove and re-insert the RAM module to simulate reseating — then document each step you took and any issues you observed during the process.

RAM Reseating Procedure

- Shut down the computer completely.

- Unplug the power cable from the computer.

- Press the power button for a few seconds to discharge remaining power.

- Open the computer/laptop back cover or case carefully.

- Locate the RAM slot/module on the motherboard.

- Release the RAM clips on both sides of the RAM module.

- Hold the RAM by its edges and remove it carefully.

- Check the RAM for dust, damage, or dirty contacts.

- Align the notch on the RAM with the slot.

- Insert the RAM firmly into the slot.

- Press both ends until the clips lock automatically.

- Close the computer case/back cover.

- Reconnect the power cable and start the computer.

- Check that the computer boots normally and recognizes the RAM.

Issues Observed

- RAM module: No visible damage.

- RAM slot: No visible damage.

- Dust: No significant dust observed.

- Installation: RAM was successfully reseated.

- Final result: PC started normally after reseating.

- Create a troubleshooting checklist for diagnosing a faulty GPU in a gaming PC, including at least 5 steps from initial symptom identification to confirming the fix.<br><br><em><strong>Hint:</strong> Think about display issues, driver checks, and physical connections.</em>

GPU Troubleshooting Checklist

- Identify the symptom

  - Check for no display, screen flickering, artifacts, crashes, or poor gaming performance.

- Check the monitor and cable

  - Make sure the HDMI/DisplayPort cable is properly connected.

  - Try another cable or monitor if available.

- Check the GPU connection

  - Turn off and unplug the PC.

  - Make sure the GPU is properly seated in the PCIe x16 slot.

- Check GPU power cables

  - Ensure the GPU's required PCIe power connectors are firmly connected to the PSU.

- Check GPU drivers

  - Open Device Manager and check Display adapters.

  - Look for warning symbols or driver errors.

  - Update or reinstall the GPU driver if required.

- Check GPU cooling

  - Check whether the GPU fans are working and remove excessive dust.

  - Ensure the GPU is not overheating.

- Test the GPU

  - Start the PC and run a game or graphics test.

  - Check whether the display works normally without crashes or artifacts.

- Confirm the fix

  - Restart the PC and test the system again.

  - If the display is stable and games run normally, the GPU problem is likely resolved.

- Simulate a storage issue by unplugging and replugging a SATA or NVMe drive (SSD/HDD) on a test system, then describe the error messages or BIOS indicators you see before and after reconnecting.

Storage Issue Simulation

- Shut down the test PC completely.

- Unplug the power cable from the PC.

- Open the computer case carefully.

- Locate the SATA SSD/HDD or NVMe SSD.

- Disconnect the storage drive:

  - SATA: Disconnect the SATA data cable.

  - NVMe: Remove the M.2 drive carefully.

- Start the PC without the storage drive.

- Check the BIOS/UEFI or boot screen.

- A typical message may be “No Boot Device Found” or “Boot Device Not Found.”

- Shut down the PC again.

- Reconnect the SATA cable or reinstall the NVMe drive securely.

- Start the PC and enter BIOS/UEFI.

- Check whether the SSD/HDD model and capacity are detected.

- Save the settings and restart.

- The operating system should boot normally after the drive is detected.

Observation

| Condition | BIOS/Boot Result |
| --- | --- |
| Drive disconnected | “No Boot Device Found” / storage drive not detected |
| Drive reconnected | SSD/HDD appears in BIOS and system boots normally |

- Given a scenario where a PC randomly restarts, list which component (RAM, GPU, storage) you would check first and explain your reasoning, then outline the specific diagnostic steps you would perform for that component.

PC Randomly Restarts: Troubleshooting Plan

Component to check first: RAM

Reason: Faulty, loose, or incompatible RAM can cause sudden restarts, crashes, or system instability. It is also easy to physically check and reseat.

Diagnostic Steps

- Shut down the PC and disconnect the power.

- Open the PC case and locate the RAM modules.

- Check the RAM seating and make sure both clips are locked.

- Remove and reseat the RAM carefully.

- If there are two or more RAM sticks, test one stick at a time.

- Try the RAM in the recommended motherboard slot.

- Start the PC and check whether the random restarts continue.

- If possible, test with a known-good compatible RAM module.

- Run a memory diagnostic test such as Windows Memory Diagnostic to check for RAM errors.

- If the PC still restarts, continue checking the GPU and storage.