# ASSIGNMENT: 5

- Draw a simple diagram showing the pin layout of an RJ-45 connector and label the color codes for the T568A and T568B Ethernet wiring standards.

- Create a table comparing the key specifications of Ethernet standards 10BASE-T, 100BASE-TX, and 1000BASE-T, including speed, cable type, maximum cable length, and connector type.

Ethernet Standards Comparison

| Specification | 10BASE-T | 100BASE-TX | 1000BASE-T |
| --- | --- | --- | --- |
| Speed | 10 Mbps | 100 Mbps | 1000 Mbps (1 Gbps) |
| Cable Type | Cat3 or Cat5 UTP | Cat5 UTP or better | Cat5e or Cat6 UTP |
| Maximum Cable Length | 100 meters | 100 meters | 100 meters |
| Connector Type | RJ-45 | RJ-45 | RJ-45 |

- Given a scenario where you need to connect two laptops directly using Ethernet, explain which cable (straight-through or crossover) you would use and why.<br><br><em><strong>Hint:</strong> Think about how transmit and receive pairs are wired in each cable type.</em>

## Scenario: Laptop-to-Laptop Connection

| Scenario | Cable to Use | Reason |
| --- | --- | --- |
| Laptop to Laptop (Direct) | Crossover Cable | Transmit and receive pairs are swapped, allowing two devices to communicate directly. |

## Types of Ethernet Cables

| Cable Type | Wiring | Use Case |
| --- | --- | --- |
| Straight-Through | Same wiring order at both ends (T568B–T568B) | Connects different devices, such as a PC to a switch. |
| Crossover | Different wiring orders at both ends (T568A–T568B) | Traditionally connects similar devices, such as PC to PC or switch to switch. |

- List three real-world devices in your home or college that use Ethernet connections, and for each, specify the likely Ethernet standard (e.g., 100BASE-TX, 1000BASE-T) they support.

Ethernet Devices at Home/College

| Device | Likely Ethernet Standard | Reason |
| --- | --- | --- |
| Home Wi-Fi Router | 1000BASE-T (Gigabit) | Modern routers often support Gigabit Ethernet for high-speed internet connectivity. |
| Desktop PC / College Lab Computer | 1000BASE-T (Gigabit) | Most modern motherboards include built-in Gigabit Ethernet ports. |
| Smart TV / Gaming Console | 100BASE-TX or 1000BASE-T | Older models may support 100 Mbps, while newer models may support 1 Gbps. |