# ASSIGNMENT: 4

- Draw a labeled diagram of a Cat6 Ethernet cable, showing the color order of wires in the T568B standard and labeling the RJ-45 connector pins.

- List three types of network cables (such as Cat5e, Cat6, and fiber optic) and for each, mention one real-world app or device (like a WiFi router, Zomato POS terminal, or IPL streaming server) where it would be used, explaining why.

| Cable Type | Real-World Use | Why |
| --- | --- | --- |
| Cat5e | Home Wi-Fi router | Affordable and supports speeds up to 1 Gbps, suitable for most home internet connections. |
| Cat6 | Zomato POS terminal | Supports high-speed data transmission and offers reliable connectivity for payment transactions. |
| Fiber Optic | IPL streaming server | Provides very high bandwidth, supports long-distance transmission, and resists electromagnetic interference. |

- Watch a YouTube video showing how to crimp an RJ-45 connector onto a network cable, then write down the step-by-step process in your own words and mention two common mistakes to avoid.

How to Crimp an RJ-45 Connector

Step-by-Step Process

| Step | Action |
| --- | --- |
| 1 | Cut the Ethernet cable to the desired length. |
| 2 | Strip about 2 cm of the outer jacket. |
| 3 | Untwist and separate the 8 wires. |
| 4 | Arrange the wires in T568B order: Orange-White, Orange, Green-White, Blue, Blue-White, Green, Brown-White, Brown. |
| 5 | Trim all wires evenly using a cable cutter. |
| 6 | Insert the wires into the RJ-45 connector, ensuring all wires reach the end. |
| 7 | Insert the connector into the crimping tool and squeeze firmly. |
| 8 | Test the cable using an Ethernet cable tester. |

Common Mistakes to Avoid

| Mistake | Why It's Bad |
| --- | --- |
| Wrong wire order | The cable may fail to establish a proper connection or transmit data correctly. |
| Wires not pushed all the way in | The wires may not make proper contact with the connector pins, causing connection failure. |

- Given the following scenario: You are setting up a gaming zone for an IPL screening event and need to connect multiple screens and consoles. Choose the appropriate cable and connector types for high-speed, reliable connections, and explain your choices.

## IPL Gaming Zone Setup – Cable & Connector Choices

| Connection Need | Cable Type | Connector | Reason |
| --- | --- | --- | --- |
| Console to Switch (LAN) | Cat6 | RJ-45 | Supports up to 10 Gbps over short distances (up to 55 m under suitable conditions) and provides reliable gaming connectivity. |
| Switch to Router / Screens | Cat6a | RJ-45 | Supports 10 Gbps up to 100 m and offers good protection against interference. |
| Long Distance (Across Hall) | Fiber Optic | LC / SC | Supports high-speed data transmission over long distances and resists electromagnetic interference. |
| Screen to Console (Video) | HDMI 2.1 | HDMI | Supports 4K resolution at up to 120 Hz with compatible devices. |
| Power Connections | Standard Power Cable | 3-Pin Plug | Supplies electrical power to screens and consoles. |

- Find and share a photo (from your home, college, or online) of any network cable or connector in use (such as behind a WiFi router or at a café's POS counter), and describe what type it is and what devices it connects.<br><br><em><strong>Hint:</strong> Look for blue or yellow cables with clear plastic ends (RJ-45) or thin round cables for fiber optics.</em>