# Assignment – 2

- List 5 common hardware issues you might encounter in a desktop PC and write the typical symptom for each (for example: 'No display on monitor' — symptom: blank screen, no POST beep).

| Hardware Issue | Typical Symptom |
| --- | --- |
| 1. RAM failure | PC may not boot, display may remain blank, or motherboard may give memory error beeps/LEDs. |
| 2. Power Supply (PSU) failure | PC does not turn on, no fans spin, and no power LEDs appear. |
| 3. Hard Drive/SSD failure | Operating system does not boot, drive is not detected, or system shows a boot-device error. |
| 4. CPU overheating | PC shuts down or restarts unexpectedly, becomes very slow, or shows high CPU temperature. |
| 5. GPU/Graphics card failure | No display, screen artifacts, driver errors, or system may fail to complete POST. |

- Given this scenario: A laptop is not charging even when plugged in. Write down a step-by-step troubleshooting plan using only basic tools (no software diagnostics), and explain what you would check at each step.

| Step | What to Check | What to Look For |
| --- | --- | --- |
| 1. Check the power outlet | Plug another device into the same outlet. | Confirm the outlet is supplying power. |
| 2. Inspect the charger | Check the adapter, cable, and charging connector. | Look for cuts, burns, bent pins, or loose connections. |
| 3. Check the charging port | Examine the laptop's charging port. | Check for dust, damage, or a loose port. |
| 4. Test charger output | Use a multimeter to measure the charger's DC output voltage. | Compare the reading with the voltage printed on the charger label. |
| 5. Check the battery connection | With the laptop powered off and unplugged, inspect the battery connection if accessible. | Look for a loose connector or damaged battery. |
| 6. Try a known-good compatible charger | Connect another charger with the correct voltage, connector, and power rating. | If it charges, the original charger is likely faulty. |
| 7. Check for physical damage | Inspect the laptop's charging circuit/port area. | Look for burnt components, loose parts, or visible damage. |
| 8. Final check | Reconnect everything and plug in the charger. | Confirm the charging indicator turns on and the battery begins charging. |

- Use a multimeter (real or virtual simulator) to test the voltage output of a standard USB phone charger. Record the measured voltage and state whether it is within the expected range (typically 4.75V to 5.25V for USB).

USB Phone Charger Voltage Test

Tool used: Digital Multimeter
Expected USB voltage: 4.75V to 5.25V DC

| Test | Measured Voltage | Expected Range | Result |
| --- | --- | --- | --- |
| USB charger output | 5.02 V DC | 4.75V – 5.25V | Within range / PASS |

Steps

- Set the multimeter to DC voltage (V⎓).

- Plug the USB charger into a power outlet.

- Touch the black probe to GND/negative of the USB connector.

- Touch the red probe to the +5V/VBUS pin.

- Read and record the voltage on the multimeter.

- Compare the reading with 4.75V–5.25V.

- A PC is randomly shutting down. Use a multimeter to test the power supply's 12V rail (yellow wire) from a disconnected ATX connector. Describe the steps and write down the voltage you find. If the voltage is below 11.5V, what hardware issue could this indicate?<br><br><em><strong>Hint:</strong> Remember to set the multimeter to DC voltage and probe between the yellow and black wires.</em>

Steps to Test PSU 12V Rail Using a Multimeter

- Turn off the PC and unplug it from the power outlet.

- Disconnect the ATX power connector from the motherboard.

- Set the multimeter to DC Voltage (V⎓).

- Turn on the PSU only when you are ready to take the measurement, following a safe PSU-testing procedure.

- Identify the yellow wire (+12V) and black wire (GND).

- Place the black probe on the black wire.

- Place the red probe on the yellow wire.

- Read the voltage shown on the multimeter.

- Example reading: 12.08V DC.

- Compare the reading with the expected 12V rail.

- If the voltage is below 11.5V, it may indicate a weak or faulty PSU.

- A faulty 12V rail can cause random shutdowns, crashes, or system instability.

Result: 12.08V = Normal 
Below 11.5V = Possible PSU problem