Day  -5 Assi

- A user reports their laptop won't power on even when plugged in. List 5 troubleshooting steps you would take to diagnose the cause, and for each step, explain what you are checking for.

| Step | What I would do | What I am checking |
| --- | --- | --- |
| 1. Check the wall outlet | Plug another working device into the same socket. | Whether the power outlet is supplying electricity. |
| 2. Inspect the charger | Check the adapter, cable, connector, and charging LED for damage. | Whether the charger is physically damaged or not receiving/supplying power. |
| 3. Check the charging port | Inspect the laptop's DC/USB-C charging port for dust, damage, or looseness. | Whether the laptop is making a proper connection with the charger. |
| 4. Perform a power reset | Disconnect the charger. If the battery is removable, remove it. Hold the power button for about 10–15 seconds, then reconnect the charger and try again. | Whether a temporary power-state problem is preventing startup. |
| 5. Check battery/power hardware | If possible, try a known-good compatible charger. Check for battery swelling or unusual heat. | Whether the battery, charger, charging circuit, or motherboard is faulty. |

- You receive a helpdesk ticket: 'My monitor is showing a blank screen, but the CPU seems to be running.' Write a step-by-step troubleshooting flow you would follow to identify and resolve the issue.<br><br><em><strong>Hint:</strong> Consider checking both hardware connections and basic display settings.</em>

Helpdesk Troubleshooting Flow — Blank Monitor

Problem: Monitor shows a blank/black screen, but the PC appears to be running.

Step-by-step flow

1. Check the monitor power

- Make sure the monitor is switched ON.

- Check the power cable and power indicator.

- If no power → check the socket, cable, or monitor power adapter.

⬇️

2. Check the monitor input/source

- Press the monitor's Input/Source button.

- Select the correct input, such as HDMI, DisplayPort, or VGA.

- If display appears → Fixed.

⬇️

3. Check the display cable

- Disconnect and firmly reconnect the HDMI/DisplayPort cable at both ends.

- Look for damaged or bent connectors.

- If available, try another known-good cable.

- If display appears → Fixed.

⬇️

4. Check where the cable is connected

- If the PC has a dedicated GPU, make sure the monitor cable is connected to the GPU's video output, not the motherboard's video port.

- If display appears → Fixed.

⬇️

5. Check the GPU

- Turn off and unplug the PC.

- Check that the GPU is properly seated in its PCIe slot.

- Check that its required PCIe power cables are connected.

- If the GPU was loose → reseat it and test again.

⬇️

6. Check RAM

- Power off the PC.

- Reseat the RAM modules.

- If there are multiple modules, test one at a time.

- Reason: RAM problems can prevent POST and result in no display.

⬇️

7. Check for POST/beep indicators

- Start the PC and listen for beep codes.

- Check motherboard diagnostic LEDs if available.

- Memory/GPU/CPU error indication → troubleshoot the indicated component.

⬇️

8. Check basic display settings

- If Windows starts but the screen remains blank, try Windows + Ctrl + Shift + B to reset the graphics driver.

- Try Windows + P and select the appropriate display mode.

- If possible, test with another monitor.

- A user complains that their wireless mouse is unresponsive, but the keyboard works fine. Describe how you would isolate whether the issue is with the mouse, the USB port, or the system settings.

Wireless Mouse Troubleshooting

Since the keyboard works, I would isolate the problem in this order:

1. Check the mouse itself

- Make sure the mouse is turned ON.

- Replace the battery or recharge it.

- Check whether the sensor light works (if applicable).

- Move the mouse and check if the cursor responds.

- If it still doesn't work → continue testing.

2. Check the wireless receiver

- Remove the mouse's USB receiver and plug it back in firmly.

- Check for physical damage.

- Test: Move the receiver to another USB port.

- If the mouse works in another port, the original USB port may be the problem.

3. Test the USB port

- Plug another working USB device, such as a USB flash drive, into the same port.

- If the other device also doesn't work → the USB port may be faulty.

- If the other device works → the port is probably fine, so continue.

4. Test the mouse on another computer

- Connect the wireless receiver to another working PC.

- Mouse works on another PC → likely a system/settings or driver issue on the original PC.

- Mouse doesn't work → likely the mouse, receiver, or battery is faulty.

5. Check system settings

- Check whether the mouse is detected in Windows Settings → Bluetooth & devices → Mouse.

- If it is a Bluetooth mouse, verify Bluetooth is ON and the mouse is paired.

- Check Device Manager → Mice and other pointing devices for errors.

- Imagine you are working on a helpdesk team for a gaming cafe. One system is overheating and shutting down during use. List 3 possible hardware causes and describe how you would check each one.<br><br><em><strong>Hint:</strong> Think about fans, dust, and thermal paste.</em>

| # | Possible Cause | How I Would Check |
| --- | --- | --- |
| 1. CPU/GPU fan not working | Power off the PC and inspect the CPU and GPU fans. Turn the PC on and check whether the fans spin normally. Also listen for unusual noise. | If a fan is not spinning, is noisy, or stops unexpectedly, the fan may need cleaning or replacement. |
| 2. Dust buildup | Turn off and unplug the PC. Open the case and inspect the CPU heatsink, GPU heatsink, case fans, and air vents. | Heavy dust can block airflow and trap heat. Clean the dust carefully using appropriate compressed air. |
| 3. Dried or poorly applied thermal paste | If fans and airflow are working but the CPU still overheats, inspect the CPU cooler mounting. If necessary, remove the cooler and check the thermal paste. | Dried, cracked, or insufficient thermal paste can prevent proper heat transfer. Clean the old paste and apply new thermal paste correctly before reinstalling the cooler. |