# ASSIGNMENT – 7

- List the IP address classes (A, B, C, D, E) and give one real-world example of a device or service that might use each class. Write your answers in a table format.

| IP Class | IP Address Range | Purpose | Real-World Example |
| --- | --- | --- | --- |
| Class A | 1.0.0.0 – 126.255.255.255 | Large networks | A large ISP network |
| Class B | 128.0.0.0 – 191.255.255.255 | Medium-sized networks | A university network |
| Class C | 192.0.0.0 – 223.255.255.255 | Small networks | A small office LAN |
| Class D | 224.0.0.0 – 239.255.255.255 | Multicasting | Video streaming to multiple devices using multicast |
| Class E | 240.0.0.0 – 255.255.255.255 | Experimental and reserved use | Research and experimental networking |

- Given the IP address 192.168.10.45 with subnet mask 255.255.255.0, calculate the network address, broadcast address, and the range of valid host addresses.<br><br><em><strong>Hint:</strong> Show your working steps for each calculation.</em>

| IP Address | 192.168.10.45 |
| --- | --- |
| Subnet Mask | 255.255.255.0 |
| Network Address | 192.168.10.0 |
| Broadcast Address | 192.168.10.255 |
| First Valid Host | 192.168.10.1 |
| Last Valid Host | 192.168.10.254 |
| Total Valid Hosts | 254 |

- Create a subnetting plan for a small food delivery startup like Zomato that needs 4 separate subnets: one for Delivery Partners, one for Restaurants, one for Customers, and one for Admins. Assign a unique subnet (with network address and subnet mask) for each group using the 10.0.0.0/24 network.

Subnetting Plan for a Food Delivery Startup

The startup has the network 10.0.0.0/24 and needs 4 separate subnets for Delivery Partners, Restaurants, Customers, and Admins.

Step 1: Calculate the new subnet mask

- Original network: 10.0.0.0/24

- Required subnets: 4

- Borrowed bits: 2, because \(2^2 = 4\)

- New prefix: /24 + 2 = /26

- New subnet mask: 255.255.255.192

- Each subnet has 64 total IP addresses and 62 usable host addresses.

Step 2: Assign the four subnets

| No. | Group | Network Address | Subnet Mask | Valid Host Range | Broadcast Address |
| --- | --- | --- | --- | --- | --- |
| 1 | Delivery Partners | 10.0.0.0/26 | 255.255.255.192 | 10.0.0.1 – 10.0.0.62 | 10.0.0.63 |
| 2 | Restaurants | 10.0.0.64/26 | 255.255.255.192 | 10.0.0.65 – 10.0.0.126 | 10.0.0.127 |
| 3 | Customers | 10.0.0.128/26 | 255.255.255.192 | 10.0.0.129 – 10.0.0.190 | 10.0.0.191 |
| 4 | Admins | 10.0.0.192/26 | 255.255.255.192 | 10.0.0.193 – 10.0.0.254 | 10.0.0.255 |

Step 3: Verify the subnetting

The block size is:

256-192=64

Therefore, the network addresses increase by 64:

- Subnet 1: 10.0.0.0

- Subnet 2: 10.0.0.64

- Subnet 3: 10.0.0.128

- Subnet 4: 10.0.0.192

Each subnet contains (2^6 = 64) total addresses, including one network address and one broadcast address.

- You are given the following scenario: A college WiFi network uses the IP range 172.16.0.0/16. The admin wants to create at least 20 subnets for different buildings. Calculate the new subnet mask and the number of hosts available per subnet.<br><br><em><strong>Constraint:</strong> Show your calculation steps and final answers.</em>

| Item | Answer |
| --- | --- |
| Original Network | 172.16.0.0/16 |
| Required Subnets | At least 20 |
| Bits Borrowed | 5 bits |
| Total Subnets Created | 2^5 = 32 |
| New Prefix | /21 |
| New Subnet Mask | 255.255.248.0 |
| Host Bits Remaining | 11 bits |
| Total IP Addresses per Subnet | 2,048 |
| Usable Hosts per Subnet | 2,046 |

- A friend sends you a message asking why their device with IP 192.168.1.300 is not connecting to the network. Write a short explanation identifying the error and what a valid IP address in that subnet should look like.

The IP address 192.168.1.300 is invalid because each octet in an IPv4 address must have a value between 0 and 255. The last octet, 300, exceeds this limit.

A valid IP address in the same subnet (assuming a /24 subnet mask of 255.255.255.0) could be 192.168.1.100, with a valid host address range of 192.168.1.1 to 192.168.1.254.