# Assignment – 1

- List three real-life scenarios where a multimeter would be used to troubleshoot a computer or smartphone issue, and explain in one line how the multimeter helps in each case.

| No. | Real-Life Scenario | How the Multimeter Helps |
| --- | --- | --- |
| 1 | Computer does not power on | It can measure the PSU output voltage to check whether the power supply is providing the required voltage. |
| 2 | Laptop charger is not working | It can measure the charger's output voltage to check whether the adapter is supplying power correctly. |
| 3 | USB port is not working | It can measure the USB port's voltage and continuity to check whether power is reaching the port. |

- Watch a short YouTube video demo of a POST card in action (search for 'POST card motherboard test'), then write a brief summary (3-4 lines) describing what the POST card displays and what problem it helped identify in the video.<br><br><em><strong>Hint:</strong> Focus on the codes shown and how they relate to hardware issues.</em>

POST Card Video Demo Summary

- A POST card is connected to the motherboard and displays POST/diagnostic codes during the computer's startup process.

- The displayed codes indicate which stage of the Power-On Self-Test (POST) the motherboard has reached.

- In the video, the POST card helps identify a hardware-related startup problem by stopping at a particular diagnostic code.

- The code helps the technician narrow down the faulty area, such as RAM, CPU, motherboard, or other hardware, instead of replacing components randomly.

- Yes, you can show 4-digit hexadecimal-style POST codes in the table. Note that actual POST/UEFI diagnostic codes vary by motherboard and manufacturer.

| POST Code | Possible Problem |
| --- | --- |
| 0000 | CPU or motherboard initialization problem |
| 0010 | CPU initialization issue |
| 0050 | Memory/RAM initialization problem |
| 0070 | Memory configuration or detection issue |
| 0090 | Storage or boot-device initialization issue |
| 00A0 | Storage initialization problem |
| 00B0 | USB/device initialization issue |
| 00D0 | PCIe/expansion device initialization issue |
| 00F0 | Boot or firmware initialization problem |
| FFFF | CPU/motherboard power or initialization failure |

- Download and install any free software diagnostic tool (such as HWMonitor, Speccy, or CrystalDiskInfo), run it on your laptop or desktop, and take a screenshot of the main dashboard showing system health details.<br><br><em><strong>Constraint:</strong> Blur or crop out any personal information before submitting.</em>

Option 1: HWMonitor (Recommended)

1. Download HWMonitor from its official website.

2. Install and open the program.

3. Wait a few seconds for it to detect your hardware.

4. Take a screenshot of the main dashboard showing:

o CPU temperature

o CPU usage

o Voltages

o Fan speeds

o Storage temperatures (if available)

5. Blur or crop out any personal information (such as your computer name or username) before submitting.

Option 2: Speccy

· Open Speccy after installation.

· Wait for the system scan to finish.

· Take a screenshot of the Summary page.

· Blur or crop any personal information.

Option 3: CrystalDiskInfo

· Open CrystalDiskInfo.

· Take a screenshot showing:

o Disk Health Status

o Temperature

o Power-on Hours

o Drive Model (optional to blur if preferred)

- Compare the use of a multimeter, POST card, and software diagnostics by creating a table with three columns (Tool, What it checks, Example Problem Detected) and fill in at least one example for each tool.

| Tool | What it checks | Example Problem Detected |
| --- | --- | --- |
| Multimeter | Checks voltage, resistance, and continuity in electrical components. | Dead PSU: Measures the PSU output voltage and detects incorrect or missing power. |
| POST Card | Checks the motherboard's startup/POST process using diagnostic codes. | RAM problem: A POST code can indicate a memory initialization or detection failure. |
| Software Diagnostics | Checks hardware status, temperature, voltage, storage health, and device errors through software. | Overheating CPU: Software such as HWMonitor can show unusually high CPU temperature. |