4-Floor LAN Network Configuration Guide

Cisco Packet Tracer • IP Addressing • VLANs • Switches • Router-on-a-Stick

# 1. Topology and assumptions

This guide documents a four-floor LAN using one router, one core switch, and four floor switches. The router uses GigabitEthernet0/0 for the 802.1Q trunk to the core switch. The example core-switch ports are Fa0/1 to the router and Fa0/2–Fa0/5 to Floor 1–Floor 4 respectively. The floor-switch uplink is assumed to be Fa0/1, based on the CDP warning observed on SW1-F1. Confirm the actual cable ports in Packet Tracer before applying commands; if a cable uses a different port, substitute that interface number in the commands.

Current PCs: Floor 1 = 6, Floor 2 = 8, Floor 3 = 6, Floor 4 = 2 (22 total). The subnet sizes allow future expansion to 14 PCs on Floor 1 and 30 PCs on Floor 2.

# 2. VLAN and subnet plan

| Floor | VLAN | Network / CIDR | Subnet mask | Default gateway | Current PC IPs | Usable hosts |
| --- | --- | --- | --- | --- | --- | --- |
| Floor 1 | 10 | 192.168.10.0/27 | 255.255.255.224 | 192.168.10.1 | 192.168.10.2–192.168.10.7 | 30 usable addresses |
| Floor 2 | 20 | 192.168.20.0/26 | 255.255.255.192 | 192.168.20.1 | 192.168.20.2–192.168.20.9 | 62 usable addresses |
| Floor 3 | 30 | 192.168.30.0/28 | 255.255.255.240 | 192.168.30.1 | 192.168.30.2–192.168.30.7 | 14 usable addresses |
| Floor 4 | 40 | 192.168.40.0/29 | 255.255.255.248 | 192.168.40.1 | 192.168.40.2–192.168.40.3 | 6 usable addresses |

Note: the router gateway address is the first usable address in each subnet. Do not assign the gateway address to a PC.

# 3. Individual PC IP addresses

| Floor | PC label | IPv4 address | Subnet mask | Default gateway |
| --- | --- | --- | --- | --- |
| Floor 1 | PC1 | 192.168.10.2 | 255.255.255.224 | 192.168.10.1 |
| Floor 1 | PC2 | 192.168.10.3 | 255.255.255.224 | 192.168.10.1 |
| Floor 1 | PC3 | 192.168.10.4 | 255.255.255.224 | 192.168.10.1 |
| Floor 1 | PC4 | 192.168.10.5 | 255.255.255.224 | 192.168.10.1 |
| Floor 1 | PC5 | 192.168.10.6 | 255.255.255.224 | 192.168.10.1 |
| Floor 1 | PC6 | 192.168.10.7 | 255.255.255.224 | 192.168.10.1 |
| Floor 2 | PC1 | 192.168.20.2 | 255.255.255.192 | 192.168.20.1 |
| Floor 2 | PC2 | 192.168.20.3 | 255.255.255.192 | 192.168.20.1 |
| Floor 2 | PC3 | 192.168.20.4 | 255.255.255.192 | 192.168.20.1 |
| Floor 2 | PC4 | 192.168.20.5 | 255.255.255.192 | 192.168.20.1 |
| Floor 2 | PC5 | 192.168.20.6 | 255.255.255.192 | 192.168.20.1 |
| Floor 2 | PC6 | 192.168.20.7 | 255.255.255.192 | 192.168.20.1 |
| Floor 2 | PC7 | 192.168.20.8 | 255.255.255.192 | 192.168.20.1 |
| Floor 2 | PC8 | 192.168.20.9 | 255.255.255.192 | 192.168.20.1 |
| Floor 3 | PC1 | 192.168.30.2 | 255.255.255.240 | 192.168.30.1 |
| Floor 3 | PC2 | 192.168.30.3 | 255.255.255.240 | 192.168.30.1 |
| Floor 3 | PC3 | 192.168.30.4 | 255.255.255.240 | 192.168.30.1 |
| Floor 3 | PC4 | 192.168.30.5 | 255.255.255.240 | 192.168.30.1 |
| Floor 3 | PC5 | 192.168.30.6 | 255.255.255.240 | 192.168.30.1 |
| Floor 3 | PC6 | 192.168.30.7 | 255.255.255.240 | 192.168.30.1 |
| Floor 4 | PC1 | 192.168.40.2 | 255.255.255.248 | 192.168.40.1 |
| Floor 4 | PC2 | 192.168.40.3 | 255.255.255.248 | 192.168.40.1 |

# 4. Router R1 configuration (router-on-a-stick)

Use this on R1 if GigabitEthernet0/0 connects to CORE-SW Fa0/1.

## R1 commands

enable
configure terminal
hostname R1
interface gigabitEthernet 0/0
 no ip address
 no shutdown
exit
interface gigabitEthernet 0/0.10
 encapsulation dot1Q 10
 ip address 192.168.10.1 255.255.255.224
exit
interface gigabitEthernet 0/0.20
 encapsulation dot1Q 20
 ip address 192.168.20.1 255.255.255.192
exit
interface gigabitEthernet 0/0.30
 encapsulation dot1Q 30
 ip address 192.168.30.1 255.255.255.240
exit
interface gigabitEthernet 0/0.40
 encapsulation dot1Q 40
 ip address 192.168.40.1 255.255.255.248
exit
end
write memory

# 5. Core switch CORE-SW configuration

Assumed ports: Fa0/1 to R1; Fa0/2 to SW1-F1; Fa0/3 to SW2-F2; Fa0/4 to SW3-F3; Fa0/5 to SW4-F4.

## CORE-SW commands

enable
configure terminal
hostname CORE-SW
vlan 10
 name FLOOR1
vlan 20
 name FLOOR2
vlan 30
 name FLOOR3
vlan 40
 name FLOOR4
exit
interface fastEthernet 0/1
 switchport mode trunk
 switchport trunk native vlan 1
 switchport trunk allowed vlan 10,20,30,40
 no shutdown
exit
interface range fastEthernet 0/2 - 5
 switchport mode trunk
 switchport trunk native vlan 1
 switchport trunk allowed vlan 10,20,30,40
 no shutdown
exit
end
write memory

# 6. Floor switch configurations

For each floor switch, the examples assume Fa0/1 is the uplink to CORE-SW. PC ports start at Fa0/2. If your actual cabling differs, change the interface numbers accordingly. The uplink must be trunking; PC-facing ports must be access ports in that floor’s VLAN.

## SW1-F1 — Floor 1, VLAN 10

enable
configure terminal
hostname SW1-F1
vlan 10
 name FLOOR1
vlan 20
 name FLOOR2
vlan 30
 name FLOOR3
vlan 40
 name FLOOR4
exit
interface fastEthernet 0/1
 switchport mode trunk
 switchport trunk native vlan 1
 switchport trunk allowed vlan 10,20,30,40
 no shutdown
exit
interface range fastEthernet 0/2 - 7
 switchport mode access
 switchport access vlan 10
 spanning-tree portfast
 no shutdown
exit
end
write memory

## SW2-F2 — Floor 2, VLAN 20

enable
configure terminal
hostname SW2-F2
vlan 10
 name FLOOR1
vlan 20
 name FLOOR2
vlan 30
 name FLOOR3
vlan 40
 name FLOOR4
exit
interface fastEthernet 0/1
 switchport mode trunk
 switchport trunk native vlan 1
 switchport trunk allowed vlan 10,20,30,40
 no shutdown
exit
interface range fastEthernet 0/2 - 9
 switchport mode access
 switchport access vlan 20
 spanning-tree portfast
 no shutdown
exit
end
write memory

## SW3-F3 — Floor 3, VLAN 30

enable
configure terminal
hostname SW3-F3
vlan 10
 name FLOOR1
vlan 20
 name FLOOR2
vlan 30
 name FLOOR3
vlan 40
 name FLOOR4
exit
interface fastEthernet 0/1
 switchport mode trunk
 switchport trunk native vlan 1
 switchport trunk allowed vlan 10,20,30,40
 no shutdown
exit
interface range fastEthernet 0/2 - 7
 switchport mode access
 switchport access vlan 30
 spanning-tree portfast
 no shutdown
exit
end
write memory

## SW4-F4 — Floor 4, VLAN 40

enable
configure terminal
hostname SW4-F4
vlan 10
 name FLOOR1
vlan 20
 name FLOOR2
vlan 30
 name FLOOR3
vlan 40
 name FLOOR4
exit
interface fastEthernet 0/1
 switchport mode trunk
 switchport trunk native vlan 1
 switchport trunk allowed vlan 10,20,30,40
 no shutdown
exit
interface range fastEthernet 0/2 - 3
 switchport mode access
 switchport access vlan 40
 spanning-tree portfast
 no shutdown
exit
end
write memory

# 7. Verification commands

## On each switch

show vlan brief
show interfaces trunk
show interfaces status
show running-config

## On router R1

show ip interface brief
show ip route

Expected router interfaces: GigabitEthernet0/0 and subinterfaces .10, .20, .30, .40 should show up/up. The routing table should show the four networks as directly connected.

# 8. Connectivity testing

From a Floor 1 PC, open Desktop → Command Prompt and test:

## Ping tests

ping 192.168.10.1
ping 192.168.20.2
ping 192.168.30.2
ping 192.168.40.2

The first ping checks the local default gateway. The other pings check routing between floor VLANs. A first ping can time out while ARP resolves; repeat the ping once. If it still fails, check the PC IP/mask/gateway, the PC access VLAN, both ends of the trunk, and that the router subinterface is up/up.

# 9. Important troubleshooting note

The earlier CDP warning reported SW1-F1 Fa0/1 with native VLAN 10 and CORE-SW Fa0/2 with native VLAN 1. The configurations in this guide standardize both ends on native VLAN 1. Also, the earlier SW1-F1 VLAN output showed Fa0/1–Fa0/7 in VLAN 10; this guide assumes Fa0/1 is the uplink and moves the six current PC ports to Fa0/2–Fa0/7. Confirm that the cables match this assumption before applying the port ranges.

# 10. Saving configuration

After verifying connectivity, save each router and switch configuration with:

## Save command

enable
write memory

# 11. Common Mistakes and What to Focus On

Use this checklist while building and troubleshooting the four-floor LAN in Cisco Packet Tracer. Most connectivity problems come from incorrect IP settings, VLAN assignment, trunk configuration, or mismatched physical ports.

| Common mistake | Why it causes a problem | What to focus on / how to check |
| --- | --- | --- |
| Wrong IP address, subnet mask, or default gateway | A PC must use an address from its floor subnet, the matching subnet mask, and that VLAN’s router gateway. | Floor 1: 192.168.10.x / 255.255.255.224 / gateway 192.168.10.1; Floor 2: 192.168.20.x / 255.255.255.192 / gateway 192.168.20.1; Floor 3: 192.168.30.x / 255.255.255.240 / gateway 192.168.30.1; Floor 4: 192.168.40.x / 255.255.255.248 / gateway 192.168.40.1. |
| Duplicate IP addresses | Two devices using the same IPv4 address can cause intermittent or failed communication. | Give every PC a unique address. Do not assign a router gateway address (.1) to a PC. |
| PC port assigned to the wrong VLAN | A PC on Floor 1 must be connected to an access port in VLAN 10; Floor 2 to VLAN 20; Floor 3 to VLAN 30; Floor 4 to VLAN 40. | Check with show vlan brief. PC-facing ports should be access ports, not trunks. |
| Uplink port configured as access instead of trunk | The link between a floor switch and CORE-SW must carry the floor VLAN across the network. | Check show interfaces trunk on both ends. Confirm the actual cable port before configuring it. |
| Native VLAN mismatch | A warning such as native VLAN 10 on one end and VLAN 1 on the other indicates inconsistent trunk settings. | Use the same native VLAN on both ends. This guide uses native VLAN 1. |
| Incorrect interface or cable assumptions | Commands applied to the wrong switch port will not fix the link and can disconnect a PC. | Use Packet Tracer cable indicators and show cdp neighbors (where supported) to identify connected ports. Verify the core-switch mapping: Fa0/1 to router, Fa0/2–Fa0/5 to floor switches only if that matches your topology. |
| Router interface or subinterface down/misconfigured | The physical router interface must be up; each VLAN subinterface needs the correct 802.1Q VLAN ID and gateway IP. | Run show ip interface brief and show ip route. Expected G0/0 and G0/0.10, .20, .30, .40 should be up/up. |
| VLAN missing on a switch or not allowed on trunk | A trunk can be up but still block a VLAN if that VLAN is not created/active or not allowed. | Check show vlan brief and show interfaces trunk on the core and relevant floor switch. Allow VLANs 10,20,30,40 where needed. |
| Forgetting to save configuration | Unsaved changes may be lost after the device restarts. | After testing, use enable then write memory on each router and switch. |
| Testing only one connection | A successful PC-to-gateway ping does not prove every VLAN or inter-floor path works. | Test the local gateway first, then test PCs on other floors. Repeat the first ping if it times out while ARP resolves. |

# 12. Recommended troubleshooting order

When a ping fails, do not change every device at once. Check in this order:

PC Desktop → IP Configuration: confirm IP address, subnet mask, and default gateway.

Check the PC-facing switch port: show vlan brief. Confirm the port is in the correct floor VLAN.

Check the floor-switch uplink and core-switch port: show interfaces trunk. Confirm both ends are trunking and allow the required VLAN.

Check the physical port mapping and link status: show interfaces status and, if supported, show cdp neighbors.

Check R1: show ip interface brief. Confirm the physical interface and required subinterface are up/up and use the correct VLAN tag/IP.

Test in order: PC to its gateway, then a PC on another floor. Fix the first failed step before continuing.

# 13. Key points to remember

VLAN separates devices into logical networks; each floor has its own VLAN and subnet.

Access ports connect end devices such as PCs; trunk ports carry multiple VLANs between switches and the router.

The router subinterfaces provide the default gateways and route traffic between the four VLANs.

A trunk showing “trunking” is not enough by itself: confirm the required VLANs are allowed and active.

Always match configuration to actual cabling. Example interface numbers in this guide are assumptions, not a substitute for checking the topology.

Make one change at a time, verify it, and save the configuration after successful tests.