# ASSIGNMENT: 3

- Draw a diagram of the OSI 7-layer model and label each layer with its correct name and a real-world example of a device or protocol that operates at that layer (e.g., Switch at Data Link, HTTP at Application).

OSI 7-Layer Model

| Layer No. | Layer Name | Protocols / Examples | Real-World Device / Use |
| --- | --- | --- | --- |
| 7 | Application | HTTP, HTTPS, FTP, SMTP, DNS | Web browsers, Gmail |
| 6 | Presentation | TLS/SSL, JPEG, GIF, MPEG, ASCII | Encryption, image and data formats |
| 5 | Session | NetBIOS, RPC, SIP | Establishing and managing sessions |
| 4 | Transport | TCP, UDP | Reliable data transfer, live streaming |
| 3 | Network | IP, ICMP, OSPF, BGP | Router |
| 2 | Data Link | Ethernet, Wi-Fi (802.11), MAC, ARP* | Switch, bridge, Network Interface Card (NIC) |
| 1 | Physical | Cables, fiber optics, radio signals | Hub, repeater, cables, connectors |

- Match the following networking scenarios to the correct OSI layer: a) Encrypting data before sending, b) Routing packets across networks, c) Converting data to electrical signals, d) Opening a web page in your browser, e) Detecting and correcting errors in frames.<br><br><em><strong>Hint:</strong> Write your answers as 'Scenario: Layer Name (Layer Number)'.</em>

| Scenario | OSI Layer Name | Layer Number |
| --- | --- | --- |
| a) Encrypting data before sending | Presentation | 6 |
| b) Routing packets across networks | Network | 3 |
| c) Converting data to electrical signals | Physical | 1 |
| d) Opening a web page in your browser | Application | 7 |
| e) Detecting and correcting errors in frames | Data Link | 2 |

- Pick any app you use daily (like WhatsApp, Instagram, or Zomato) and describe how at least three different OSI layers are involved when you send or receive data in that app.

App Chosen: WhatsApp

| Step | What Happens | OSI Layer Name | Layer Number |
| --- | --- | --- | --- |
| 1 | You type a message and tap Send; WhatsApp handles the messaging process. | Application | 7 |
| 2 | The message is encrypted end-to-end so only the intended participants can read it. | Presentation* | 6 |
| 3 | TCP divides data into segments and provides reliable delivery when TCP is used. | Transport | 4 |
| 4 | IP routes packets across networks toward their destination. | Network | 3 |
| 5 | Wi-Fi or mobile networks transmit data as radio signals. | Physical | 1 |

- Given this scenario: You are unable to access a website, but you can successfully ping its IP address. Identify which OSI layer(s) might be causing the problem and explain your reasoning.

Answer:

If you cannot access a website but can successfully ping its IP address, the problem may be occurring at the Application Layer (Layer 7), Transport Layer (Layer 4), or Presentation Layer (Layer 6).

| OSI Layer | Possible Problem | Reason |
| --- | --- | --- |
| Application (Layer 7) | DNS or web server issue | The browser may be unable to resolve the domain name, or the web server may not respond correctly. |
| Transport (Layer 4) | TCP port blocked | Ports 80 (HTTP) or 443 (HTTPS) may be blocked by a firewall. |
| Presentation (Layer 6) | TLS/SSL error | HTTPS encryption or certificate problems may prevent the website from loading. |