# ASSIGNMENT – 03

- List the key differences between IPv4 and IPv6 addressing by creating a two-column table with at least 4 points for each.

Differences Between IPv4 and IPv6

| IPv4 | IPv6 |
| --- | --- |
| 1. Uses 32-bit addresses. | 1. Uses 128-bit addresses. |
| 2. Written in decimal format, separated by dots. Example: 192.168.1.1 | 2. Written in hexadecimal format, separated by colons. Example: 2001:db8::1 |
| 3. Provides approximately 4.3 billion possible addresses. | 3. Provides a vastly larger address space of \(2^{128}\) addresses. |
| 4. Supports broadcast communication. | 4. Does not use broadcast; it uses multicast and anycast instead. |
| 5. Routers may fragment packets when needed. | 5. Only the sending host performs packet fragmentation; routers do not fragment IPv6 packets. |
| 6. Header size is variable, from 20 to 60 bytes. | 6. The basic header has a fixed size of 40 bytes. |
| 7. NAT is commonly used to conserve public IPv4 addresses. | 7. NAT is generally not required because of the large address space. |
| 8. IPsec support is optional. | 8. IPsec support is standardized in IPv6, but its use is not mandatory in every deployment. |

- Given the following scenario: You have 4 devices (laptop, phone, smart TV, and printer) connected to your home WiFi. Assign a unique static IPv4 address to each device using the 192.168.10.0/24 network, and write down the IP, subnet mask, and default gateway for each device.

Static IPv4 Addressing Plan for a Home Wi-Fi Network

Given network: 192.168.10.0/24

- Subnet mask: 255.255.255.0

- Default gateway: 192.168.10.1 (assumed to be the home router's IP address)

- Network address: 192.168.10.0

- Broadcast address: 192.168.10.255

IP Address Assignment Table

| Device | Static IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Laptop | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Phone | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| Smart TV | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| Printer | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |

Explanation

- Unique IP addresses: Each device has a different IP address to prevent address conflicts.

- Subnet mask: 255.255.255.0 means all devices belong to the same /24 network.

- Default gateway: 192.168.10.1 is the assumed router address used to communicate with devices outside the local network.

- Static addressing: These addresses remain assigned to the devices until manually changed.

- Calculate the number of valid subnets and hosts per subnet if you use a /27 CIDR subnet mask on a 192.168.1.0 network. List the first three valid subnet address ranges (network address to broadcast address) and the usable host IPs in each.

| Item | Answer |
| --- | --- |
| CIDR Prefix | /27 |
| Subnet Mask | 255.255.255.224 |
| Bits Borrowed (from /24) | 3 bits |
| Total Subnets | 8 |
| Total IP Addresses per Subnet | 32 |
| Usable Hosts per Subnet | 30 |
| Block Size | 32 |

- You are given the IP address 10.0.5.17 with subnet mask 255.255.255.0. Determine the network address, broadcast address, and the range of valid host addresses for this subnet.<br><br><em><strong>Hint:</strong> Convert the subnet mask to CIDR notation and use bitwise AND for network address calculation.</em>

| Item | Answer |
| --- | --- |
| IP Address | 10.0.5.17 |
| Subnet Mask | 255.255.255.0 |
| CIDR Notation | /24 |
| Network Address | 10.0.5.0 |
| Broadcast Address | 10.0.5.255 |
| First Valid Host | 10.0.5.1 |
| Last Valid Host | 10.0.5.254 |
| Total Usable Hosts | 254 |

- Use an online subnet calculator tool (such as https://www.subnet-calculator.com/) to verify your answer for the previous task. Take a screenshot of your calculation and write one line explaining if your manual answer matched the tool's output.

| Item | Calculator Result |
| --- | --- |
| IP Address | 10.0.5.17 |
| Subnet Mask | 255.255.255.0 |
| CIDR Notation | /24 |
| Network Address | 10.0.5.0 |
| Broadcast Address | 10.0.5.255 |
| First Valid Host | 10.0.5.1 |
| Last Valid Host | 10.0.5.254 |
| Total Usable Hosts | 254 |