ASSIGNMENT – 05

- Connect your laptop and a friend's phone to the same Wi-Fi network, then use the ping command to check if both devices can communicate with each other.<br><br><em><strong>Hint:</strong> On Windows, use 'ping [IP address]' in Command Prompt; on Android, try a ping app or use Termux.</em>

C:\Users\visha>ping 192.168.29.170

Pinging 192.168.29.170 with 32 bytes of data:

Reply from 192.168.29.170: bytes=32 time=594ms TTL=64

Reply from 192.168.29.170: bytes=32 time=415ms TTL=64

Reply from 192.168.29.170: bytes=32 time=217ms TTL=64

Reply from 192.168.29.170: bytes=32 time=29ms TTL=64

Ping statistics for 192.168.29.170:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 29ms, Maximum = 594ms, Average = 313ms

- Run the traceroute (tracer on Windows or traceroute on Mac/Linux) command to www.youtube.com from your device and note down the number of hops it takes to reach the destination.

C:\Users\visha>tracert www.youtube.com

Tracing route to www.youtube.com [142.251.152.4]

over a maximum of 30 hops:

1     1 ms     1 ms     1 ms  reliance.reliance [192.168.29.1]

2     2 ms     3 ms     3 ms  10.216.248.1

3    11 ms    11 ms    11 ms  172.31.3.224

4    11 ms    13 ms    13 ms  192.168.103.190

5    12 ms    19 ms    19 ms  172.26.108.134

6    12 ms    12 ms    12 ms  172.26.108.146

7    14 ms    11 ms    11 ms  192.168.188.62

8     *        *        *     Request timed out.

9     *        *        *     Request timed out.

10    26 ms    29 ms    23 ms  142.251.152.4

Trace complete.

- Disconnect one device from the Wi-Fi, then try pinging it again from your laptop and describe the error message you receive.

Pinging 192.168.29.170 with 32 bytes of data:

General failure.

General failure.

General failure.

General failure.

Ping statistics for 192.168.29.170:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss),

- Create a simple table listing the IP addresses of all devices connected to your home Wi-Fi (use 'arp -a' on Windows), and identify which device belongs to which person or gadget.

| No. | IP Address | Device | Person/Use |
| --- | --- | --- | --- |
| 1 | 192.168.29.1 | Router | Home Wi-Fi |
| 2 | 192.168.29.139 | Laptop | Me |
| 3 | 192.168.29.170 | Phone | Friend |
| 4 | 192.168.29.xxx | Phone/TV/etc. | Family member |

- Write a one-page report explaining step-by-step how you set up the network, tested connectivity using ping and traceroute, and what results or issues you observed.<br><br><em><strong>Hint:</strong> Include screenshots or command outputs if possible.</em>

Network Setup and Connectivity Testing Report

Introduction

The purpose of this practical was to set up and test a basic network connection using Wi-Fi. The network was tested by checking IP configuration, testing communication between devices using the ping command, tracing the network path using tracert, and viewing connected devices using arp -a.

Step 1: Check Network Configuration

First, I connected my laptop to the home Wi-Fi network. I opened Command Prompt and used the ipconfig command to check the network configuration. My laptop received the following details:

- IPv4 Address: 192.168.29.139

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.29.1

These values confirmed that my laptop was connected to the Wi-Fi network.

[Insert Screenshot of ipconfig output here]

Step 2: Set a Static IP

For testing, I changed the laptop's IP configuration from automatic DHCP to a static IP. I used:

- IP Address: 192.168.29.50

- Subnet Prefix: 24

- Gateway: 192.168.29.1

- DNS: 8.8.8.8

After testing, I changed the settings back to Automatic (DHCP) to avoid network problems.

Step 3: Test Connectivity Using Ping

I connected my friend's phone to the same Wi-Fi network and found its IP address as 192.168.29.170. From the laptop, I used:

ping 192.168.29.170

When the phone was connected to Wi-Fi, the laptop could communicate with it. I then disconnected the phone from Wi-Fi and ran the same ping command again. The result was:

General failure.

The statistics showed:

Sent = 4, Received = 0, Lost = 4 (100% loss)

This happened because the phone was disconnected from the network and could not respond to the ping request.

[Insert Screenshot of ping output here]

Step 4: Test Using Traceroute

I used the Windows tracert command to check the path from my laptop to YouTube:

tracert www.youtube.com

The command displayed several hops between my laptop and the destination. The number of hops was recorded from the last numbered line of the output.

[Insert Screenshot of tracert www.youtube.com output here]

Step 5: Check Network Devices

Finally, I used the following command to view devices known to my laptop on the local network:

arp -a

The command displayed IP addresses and corresponding physical (MAC) addresses. I used the IP addresses to identify devices such as the router, laptop, and phones.

[Insert Screenshot of arp -a output here]

Conclusion

The practical helped me understand basic network configuration and connectivity testing. I successfully checked IP settings, tested device communication using ping, traced the route to YouTube using tracert, and viewed local network devices using arp -a. The main issue observed was 100% packet loss and “General failure” when attempting to ping a phone that had been disconnected from Wi-Fi.