TOPS NETWORK SEGMENTATION
AND ACCESS CONTROL

Cisco Packet Tracer Practical Documentation

Topic: Router Configuration, VLANs, Inter-VLAN Routing, and ACLs

Student Name: ______________________________

Enrollment / Roll No.: ______________________

Class / Division: ____________________________

Submission Date: ___________________________

# 1. Aim

To design and configure a departmental network for TOPS using one router, one Layer-2 switch, three VLANs, router-on-a-stick inter-VLAN routing, and Access Control Lists (ACLs) to enforce the required communication policy.

# 2. Software and Devices Required

Cisco Packet Tracer

1 router with a FastEthernet0/0 interface (as shown in the supplied topology)

1 Cisco 2960 switch

3 PCs: PC0 (Student), PC1 (Faculty), and PC2 (Admin)

Copper straight-through Ethernet cables

# 3. Network Topology and Port Mapping

Use the supplied topology: Router0 FastEthernet0/0 connects to Switch0 FastEthernet0/1. PC0 (Student) connects to Switch0 Fa0/2, PC1 (Faculty) to Fa0/3, and PC2 (Admin) to Fa0/4.

Topology layout:

Router0 Fa0/0 ↔ Switch0 Fa0/1 (802.1Q trunk)

PC0 Student ↔ Switch0 Fa0/2 (access VLAN 10)

PC1 Faculty ↔ Switch0 Fa0/3 (access VLAN 20)

PC2 Admin ↔ Switch0 Fa0/4 (access VLAN 30)

# 4. IP Addressing Plan

| Device / Department | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 — Student / VLAN 10 | 192.168.10.2 | 255.255.255.0 | 192.168.10.1 |
| PC1 — Faculty / VLAN 20 | 192.168.20.2 | 255.255.255.0 | 192.168.20.1 |
| PC2 — Admin / VLAN 30 | 192.168.30.2 | 255.255.255.0 | 192.168.30.1 |

# 5. Access Control Policy

The required source-to-destination access matrix is:

| Source → Destination | Students | Faculty | Admin |
| --- | --- | --- | --- |
| Students | Allow | Deny | Deny |
| Faculty | Allow | Allow | Deny |
| Admin | Deny | Allow | Allow |

Note: The ACL examples in this document are designed to demonstrate the required directional policy for ICMP ping testing. Stateless ACLs do not track sessions; permitted return traffic for other applications may require additional protocol-specific rules or a stateful firewall.

# 6. Step-by-Step Configuration

## Step 1 — Build the topology

Open Cisco Packet Tracer.

Place one router, one Cisco 2960 switch, and three PCs on the workspace.

Rename the devices if desired: Router0, Switch0, PC0-Student, PC1-Faculty, and PC2-Admin.

Connect Router0 Fa0/0 to Switch0 Fa0/1.

Connect PC0 to Switch0 Fa0/2, PC1 to Fa0/3, and PC2 to Fa0/4.

Use copper straight-through cables.

## Step 2 — Create VLANs and configure switch ports

Open Switch0 → CLI. Enter the commands one line at a time:

enable
configure terminal

vlan 10
 name STUDENTS
exit

vlan 20
 name FACULTY
exit

vlan 30
 name ADMIN
exit

interface fastEthernet 0/2
 switchport mode access
 switchport access vlan 10
 no shutdown
exit

interface fastEthernet 0/3
 switchport mode access
 switchport access vlan 20
 no shutdown
exit

interface fastEthernet 0/4
 switchport mode access
 switchport access vlan 30
 no shutdown
exit

interface fastEthernet 0/1
 switchport mode trunk
 switchport trunk allowed vlan 10,20,30
 no shutdown
exit

end
write memory

Verify the switch configuration:

show vlan brief
show interfaces trunk

## Step 3 — Configure router-on-a-stick

Open Router0 → CLI. The commands below match the FastEthernet0/0 interface shown in the topology.

enable
configure terminal

interface fastEthernet 0/0
 no ip address
 no shutdown
exit

interface fastEthernet 0/0.10
 encapsulation dot1Q 10
 ip address 192.168.10.1 255.255.255.0
exit

interface fastEthernet 0/0.20
 encapsulation dot1Q 20
 ip address 192.168.20.1 255.255.255.0
exit

interface fastEthernet 0/0.30
 encapsulation dot1Q 30
 ip address 192.168.30.1 255.255.255.0
exit

end
write memory

## Step 4 — Configure PC IP addresses

For each PC, open Desktop → IP Configuration and enter the IP address, subnet mask, and default gateway from Section 4. Leave DNS blank unless your instructor requires it.

## Step 5 — Configure ACLs

These ACLs are applied inbound on each department subinterface. They block prohibited source-to-destination traffic and include ICMP echo-reply exceptions for the permitted ping directions in the stated matrix.

enable
configure terminal

ip access-list extended STUDENTS-IN
 permit icmp 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255 echo-reply
 deny ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255
 deny ip 192.168.10.0 0.0.0.255 192.168.30.0 0.0.0.255
 permit ip any any
exit

ip access-list extended FACULTY-IN
 permit icmp 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255 echo-reply
 deny ip 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255
 permit ip any any
exit

ip access-list extended ADMIN-IN
 deny ip 192.168.30.0 0.0.0.255 192.168.10.0 0.0.0.255
 permit ip any any
exit

interface fastEthernet 0/0.10
 ip access-group STUDENTS-IN in
exit

interface fastEthernet 0/0.20
 ip access-group FACULTY-IN in
exit

interface fastEthernet 0/0.30
 ip access-group ADMIN-IN in
exit

end
write memory

Important: This configuration is a basic lab example for the specified ping-direction tests, not a fully stateful security policy for every application. In particular, test the permitted directions in Packet Tracer and inspect ACL counters. If broader application-level access must be enforced, use stateful inspection or design explicit return-traffic rules.

# 7. Connectivity and ACL Testing

Open each PC → Desktop → Command Prompt. Run the following commands from the indicated source PC.

| Source PC | Command | Expected Result |
| --- | --- | --- |
| PC0 Student | ping 192.168.10.2 (self test; optional) | Local stack test; not a VLAN test |
| PC0 Student | ping 192.168.20.2 | Deny |
| PC0 Student | ping 192.168.30.2 | Deny |
| PC1 Faculty | ping 192.168.10.2 | Allow |
| PC1 Faculty | ping 192.168.20.2 (self test; optional) | Local stack test; not a VLAN test |
| PC1 Faculty | ping 192.168.30.2 | Deny |
| PC2 Admin | ping 192.168.10.2 | Deny |
| PC2 Admin | ping 192.168.20.2 | Allow |
| PC2 Admin | ping 192.168.30.2 (self test; optional) | Local stack test; not a VLAN test |

Because the topology has one PC per department, it cannot test same-department communication between two different PCs. Add a second PC to each VLAN if you need to demonstrate intra-department ping connectivity.

The first ping can fail while ARP resolves the destination MAC address. Repeat the ping and record the final result.

# 8. Verification Commands

## On Router0

show ip interface brief
show ip route
show access-lists
show running-config

## On Switch0

show vlan brief
show interfaces trunk
show running-config

Confirm the router subinterfaces are up/up, VLAN 10/20/30 contain the correct access ports, Fa0/1 is trunking, and ACL counters increase when the tests are run.

# 9. Save and Submit

In Packet Tracer, choose File → Save As.

Save the project as TOPS_Network_Segmentation.pkt.

Capture a screenshot of the complete topology.

Capture the switch VLAN and trunk verification outputs.

Capture the router subinterface and ACL configuration outputs.

Capture successful and blocked ping results.

Submit this DOCX documentation and the Packet Tracer PKT file.

# 10. Result / Conclusion

The TOPS network is designed with three separate VLANs: Students (VLAN 10), Faculty (VLAN 20), and Admin (VLAN 30). Router-on-a-stick provides inter-VLAN routing, while extended ACLs enforce the specified communication restrictions. The final practical result must be confirmed by running the ping tests and checking ACL counters in Cisco Packet Tracer.